SCHOOL OF EDUCATION AND BUSINESS STUDIES

BACHELOR OF INFORMATION COMMUNICATION TECHNOLOGY

BCT 117: ELECTRICITY AND MAGNETISM 1

Jan-Apr 2026 End of Semester Exams   Time: 2 Hours Lecturer: Dr Muchiri

Useful consonants:

Electron charge, e = 1.6 × 10-19 C

Permeability of free space, μ0 = 4π x 10-7 H/m

Permittivity of free space, = 8.85× 10-12 F/m

Electron mass = 9.1×10-31 kg

Mass of a proton/neutron, =1.67× 10-27 kg

Coulombs constant = 9.0 ×109 N.m2/C2

INSTRUCTIONS: ANSWER QUESTION ONE AND ANY OTHER TWO QUESTIONS

QUESTION ONE (30 MARKS)

- Differentiate between ohmic and non-ohmic conductors based on their voltage–current relationship.                                                                                                             (2 marks

- A current of 5A flows in the winding of an electric motor, the resistance of the winding being 100 Ω. Calculate the power dissipated by the coil.                                       (3 marks)

- Calculate the resistance of a 2 km length of aluminum overhead power cable if the cross-sectional area of the cable is 100 mm2. Take the resistivity of aluminum to be 0.03 × 10−6 Ω m

- State and briefly explain two factors that determine the capacitance of a parallel-plate capacitor                                                                                                 (2 marks)

- In Fig. 1, find the equivalent capacitance of the combination. Give that C1 is 10.0 μF, C2 is 5.00 μF, and C3 is 4.00 μF. 	(3 marks)

Fig 1

- State the Coulomb’s Law  	(1 mark)

- A particle of charge 3.0 ×10 -6 C is 12.0 cm distant from a second particle of charge 1.5×10 -6 C. Calculate the magnitude of the electrostatic force between the particles.       (3 marks)

- Define Ampère’s Law. 							        (1 marks)

- A long straight wire carries a current of 5.0 A. Using Ampère’s law, calculate the magnetic field at a point 0.10 m from the wire. 					        (2 marks)

- (i) State the Kirchhoff’s current and voltage laws. 	(2 marks)

(ii) Calculate the current density J given that the drift velocity v = 20 m/s, and charge density, n = 2000 electrons/m3. 	(3 marks)

- A magnetic field strength of 5000 A/m is applied to a circular magnetic circuit of mean diameter 250 mm. If the coil has 500 turns calculate the current in the coil.	 (3 marks)

- Define the following terms

    - Magnetic flux 	density						         (1 mark)

    - Inductance 						                     (1 mark)

- A magnetic pole has a rectangular section having dimensions 400 mm by 200 mm. If the total flux emerging from the pole is 200 μWb, determine the flux density               (3 marks)

QUESTION TWO (15 MARKS)

- State Ohms law 								       (1 mark)

- A copper wire of length 2.0 m and cross-sectional area 1.2 × 10⁻⁶ m² is connected across a 6.0 V battery. The resistivity of copper is 1.7 × 10⁻⁸ Ω·m. Calculate

- The resistance of the wire. 						      (3 marks)

- The current flowing through the wire.                                                     (2 marks)

- The electrical conductivity of copper.                                                     (1 mark)

- A solenoid with 100 turns per cm carries a current of 5 A. Calculate

- Magnetic field strength, H inside the solenoid                                        (2 marks)

- Magnetic flux B inside the solenoid when filled with dry air                (3 marks)

- The magnetic flux B if the solenoid is filled with a material whose magnetic susceptibility Xm = 99                                                                             (3 marks)

QUESTION THREE (15 MARKS)

- (i) Show that Coulomb’s law is consistent with Gauss’s law	(2 marks)

(ii) Using Gauss’ law, derive an expression for the capacitance of a cylindrical capacitor consisting of two coaxial cylinders of length L, with inner radius a and outer radius b. Hence show that 	 (4 marks)

- A parallel plate capacitor has plates with dimensions 3 cm by 4 cm separated by 2 mm. The plates are connected across a 60 v battery. Calculate

(i) The capacitance of the capacitor. 	(3 marks)

(iii) The stored energy in the capacitor 	(3 marks)

- Calculate its capacitance of the earth assuming that it is an isolated sphere of radius 6370 km.                                                                                                                            (3 marks)

QUESTION FOUR (15 MARKS)

- (i) Define electric potential 	(1 mark)

(ii) A point charge of magnitude 1.1×10−9 C is located in free space. Calculate the electric potential at a point 100 mm away from the charge.	 (3 marks)

- Explain three advantages of a Cathode Ray Oscilloscope (CRO) compared to an ordinary voltmeter when measuring electrical signals 	(6 marks)

- A current of 5 A is passing through a 1000 turn coil wound on a circular magnetic circuit of radius 120 mm. calculate

i. The magneto motive force 	(3 marks)

ii. The magnetic field strength 	(2 marks)

QUESTION FIVE (15 MARKS)

- (i) State the Lenz’s and  Faraday’s laws of electromagnetic induction 	        (2 marks)

(ii) Explain three factors that affect the force on a current carrying conductor     (3 marks)

- Using Faraday's law of electromagnetic induction, derive an expression for the induced emf in the loop as it enters the magnetic field. 				        (3 marks)

- A rectangular conducting loop of resistance 2.0 Ω has dimensions 0.30 m × 0.20 m. The loop moves with a constant velocity of 4.0 m/s perpendicular to a uniform magnetic field of magnitude 0.8 T. The magnetic field exists only within a region of finite width such that the loop is partly inside the field. Calculate

  - The magnitude of the induced emf as the loop enters the field.            (3 marks)

  - The induced current in the loop and state its direction using Lenz's law (4 marks)